Autonomous Drone Obstacle Avoidance Using Monocular Depth Estimation

Technical Reference. Commands, Formulae and MiDaS Integration

| Project | Autonomous Drone Obstacle Avoidance Using Monocular Depth Estimation |
| --- | --- |
| Team | Siddharth Lama, Manit Aravind, Sahil Asopa, Riyaan Sheth |
| Simulator | MuJoCo 3.13, Python 3.13, macOS on Apple Silicon |
| Depth model | MiDaS-small v2.1, 21.3 million parameters, used as published |
| Repository | /Users/sid/drone-sim |
| Date | 11 September 2026 |

1. Contents

| Sr. No. | Section | Page |
| --- | --- | --- |
| 1 | Contents | 1 |
| 2 | How to run | 1 |
| 3 | Conventions and notation | 4 |
| 4 | Camera geometry | 5 |
| 5 | Depth map to free space | 5 |
| 6 | Heading selection and speed | 7 |
| 7 | Safety layer and wing guard | 7 |
| 8 | All round obstacle memory | 8 |
| 9 | Navigation and motion | 8 |
| 10 | Course generation | 8 |
| 11 | MiDaS integration | 9 |
| 12 | Measured values | 11 |
| 13 | Parameter reference | 12 |

2. How to run

All commands are run from the repository root. The virtual environment holds MuJoCo, NumPy, OpenCV, PyTorch and the MiDaS dependencies.

2.1 The simulator on ground truth depth

Depth comes straight from the MuJoCo depth buffer. Nothing is estimated and no neural network is loaded. This is the ceiling arm of the ablation.

cd /Users/sid/drone-sim

.venv/bin/python fly.py --seed 0

Controls. Click the map panel to add a station and right click to remove the nearest one. Press N to engage autonomous flight and C to clear the route. The arrow keys fly manually, a and d strafe, r and f change altitude, space resets, and q quits.

2.2 The simulator on MiDaS estimated depth

The same aircraft and the same policy, but the depth map now comes from the network rather than the simulator. The first launch fits the scale and offset once and caches them, so later launches start immediately.

cd /Users/sid/drone-sim

.venv/bin/python fly.py --seed 0 --midas

The middle column changes to three panels. The camera image, the MiDaS depth the aircraft is flying on, and the true depth underneath as a reference the aircraft never sees. The heads up block reports the live absolute relative error and the fraction of pixels within a factor of 1.25.

| Option | Effect |
| --- | --- |
| --midas | Fly on estimated depth instead of simulator truth |
| --align frozen | Scale and offset fitted once on a calibration set. The honest arm |
| --align oracle | Refit per frame against true depth. An upper bound, not achievable in flight |
| --seed N | Choose the course. The same integer always gives the same layout |
| --obstacles N | Obstacle count. Default 9 |
| --room M | Room side in metres. Default 10 |
| --flat | Untextured world, for comparing visual realism |
| --show-hull | Tint the collision envelope so you can see what actually collides |
| --snapshot PATH | Fly a route headlessly and save one composite frame, then exit |

2.3 Headless evaluation and the ablation

Scoring runs without opening a window, which is how every number in the report is produced.

# score many seeds on ground truth depth

.venv/bin/python run_route.py --seeds 100-149 --stations 4

# the three arm ablation: perfect depth, MiDaS depth, no depth

.venv/bin/python ablation.py --seeds 100-149

# depth quality and speed of the model on simulator frames

.venv/bin/python bench_midas.py

# regenerate the wall, floor and obstacle textures

.venv/bin/python textures.py

# rebuild the airframe mesh from the team's CAD

.venv/bin/python convert_cad.py cad/

3. Conventions and notation

| Symbol | Meaning | Units |
| --- | --- | --- |
| W, H | Camera image width and height in pixels. 320 by 240 | pixels |
| i, j | Image column index and row index | index |
| D[j,i] | Depth at a pixel, measured along the camera axis, not radial | metres |
| theta_i | Yaw offset the column i corresponds to. Positive is left | radians |
| psi | Aircraft heading in the world frame | radians |
| beta | Bearing to the next station in the body frame. Positive is left | radians |
| v, omega | Commanded forward speed and yaw rate | m/s, rad/s |
| b | Half the wingspan plus margin. 0.55 m | metres |
| R | Range beyond which a direction counts as fully open. 8 m | metres |

Two conventions matter and both were verified rather than assumed. MuJoCo returns planar depth, meaning distance along the camera axis, which was confirmed by pointing the camera at a flat wall and checking that every column reported the same value. And the camera is mounted with image right along the body negative y axis, so a column to the right of centre corresponds to a right turn, which is where the minus sign in the yaw offset comes from.

4. Camera geometry

The vertical field of view is set on the camera. The horizontal one follows from the aspect ratio.

hfov = 2 * atan( tan(vfov / 2) * W / H )

Each image column maps to a yaw offset, and each row to a height gain used later to discard floor and ceiling.

ndc_x[i]   = (i + 0.5) / W * 2 - 1          # -1 at the left edge, +1 at the right

theta[i]   = -atan( ndc_x[i] * tan(hfov / 2) )

ndc_y[j]   = 1 - 2 * (j + 0.5) / H          # +1 at the top, -1 at the bottom

row_gain[j]= ndc_y[j] * tan(vfov / 2)

5. Depth map to free space

5.1 Discarding floor and ceiling

A pixel at depth D on row j sits at this height relative to the camera. Only surfaces near the aircraft's own flying height can be struck, so everything outside a band is thrown away. Without this step the floor dominates every column and the policy cannot tell open space from an obstacle.

height[j,i] = D[j,i] * row_gain[j]

keep if  |height[j,i]| < z_band                 z_band = 0.35 m

raw[i]  = clip( min over kept rows of D[.,i],  0,  R )

5.2 Angular shadow casting

A gap only counts as a gap if the aircraft fits through it. Every obstacle blocks a wedge of headings whose half width depends on that obstacle's own range. Sizing the wedge from the candidate heading's range instead is a sign error that makes close obstacles poison headings they do not really block.

k[j]          = atan( b / max(raw[j], 0.35) )

shadowed[i,j] = |theta[i] - theta[j]| < k[j]

free[i]       = min over shadowed j of raw[j],   otherwise R

The clearance directly ahead, which the safety layer uses, is the worst value inside a narrow forward wedge.

min_front = min of free[i] over |theta[i]| < 12 degrees

6. Heading selection and speed

Every candidate heading is scored on three terms. Open space, alignment with the station, and a penalty for abandoning the heading already chosen. The third term exists because two gaps of near equal value otherwise make the aircraft alternate between them at loop rate and average its heading to zero.

align[i] = cos( clip( theta[i] - beta, -pi, pi ) )

stay[i]  = |theta[i] - psi_cmd| / hfov

score[i] = free[i] / R  +  w_goal * align[i]  -  w_commit * stay[i]

best     = theta[ argmax score ]

The chosen heading is smoothed over time with an exponential moving average. This is what lets the loop tolerate a depth map that flickers frame to frame, which matters much more once MiDaS supplies the depth.

a       = exp( -dt / tau )                      tau = 0.25 s

psi_cmd = a * psi_cmd + (1 - a) * best

Speed falls as the space ahead shrinks, but never below a floor. Allowing it to decay smoothly to zero lets the aircraft park itself just outside the stop threshold forever without ever declaring itself blocked.

throttle = clip( (min_front - d_stop) / (d_clear - d_stop), 0, 1 )

throttle = max( throttle, throttle_min )

v        = v_max * throttle

* clip( cos psi_cmd, 0.25, 1 )        # slow down while turning hard

* clip( cos beta,    0.20, 1 )        # slow down if the station is behind

omega    = clip( 2.2 * psi_cmd, -omega_max, omega_max )

climb    = clip( 1.5 * altitude_error, -c_max, c_max )

7. Safety layer and wing guard

The safety layer is the guardrail and contains no learned component. It is the reason the system can be argued to be safe at all.

blocked = min_front < d_stop                    d_stop = 1.2 m

if blocked:

v     = 0,  or -v_escape after escape_after seconds

omega = turn_dir * omega_max * 0.85         # direction latched once, then held

The turn direction is chosen once when the block begins, using mean free space on each side with any side the wing cannot sweep through removed from consideration. Re-deciding every frame is what produced the visible left and right jitter.

left_free  = mean of free[i] for theta[i] > 0

right_free = mean of free[i] for theta[i] < 0

penalise a side whose wing clearance is below  b + 0.25

turn_dir   = +1 if left_free >= right_free else -1

The wing guard damps a rotation that would sweep the wingtip into something the memory still holds. It only damps. Reversing the latch here recreates the jitter it exists to prevent.

reach     = b + 0.25

wing_risk = left_clear  if omega > 0  else right_clear

if wing_risk < reach:   omega = omega * 0.15

Hysteresis on the stop threshold was implemented and measured at release margins of 0.25, 0.45 and 0.70 metres. Every setting scored worse than no hysteresis, so the parameter exists but defaults to zero.

8. All round obstacle memory

The camera sees a wedge of about 116 degrees. The wingtip sweeps a full circle. A coarse memory of 72 bearing bins keeps obstacles that have left the frame from being forgotten at the moment they matter most. It is ego motion compensated and it decays, so nothing is localised and no map is built.

# carry each remembered obstacle through last step's motion

x  = d * cos(phi) - v * dt

y  = d * sin(phi)

xr = cos(-omega*dt) * x - sin(-omega*dt) * y

yr = sin(-omega*dt) * x + cos(-omega*dt) * y

d' = hypot(xr, yr)          phi' = atan2(yr, xr)

# forget slowly, then overwrite whatever the camera can currently see

d' = min( R,  d' + forget * dt )               forget = 0.15 m/s

The decay rate is the single most influential parameter in the whole policy. Sweeping it between 0.15 and 3.0 metres per second moved collision free completion from 9 out of 12 down to 4 out of 12 on the tuning seeds.

9. Navigation and motion

wrap(a)  = (a + pi) mod 2*pi - pi

beta     = wrap( atan2(y_station - y, x_station - x) - psi )

arrived  when |station - position| < 0.7 m

# body frame command to a world frame step

step = [ cos(psi)*v - sin(psi)*s,   sin(psi)*v + cos(psi)*s,   climb ] * dt

# free joint orientation from heading

quat = [ cos(psi/2), 0, 0, sin(psi/2) ]

Motion is kinematic with collision rejection. The proposed move is written to the model and the contact count is read back. If the full move collides, rotation alone is tried, then translation alone. Refusing everything leaves a wedged aircraft unable to turn away, and because its view never changes it reissues the identical command forever.

if clear(pos + step, psi + omega*dt):  accept both

elif clear(pos, psi + omega*dt):       accept the rotation only

elif clear(pos + step, psi):           accept the translation only

else:                                  hold position and count a contact

10. Course generation

Courses come from an integer seed rather than a stored file, so the dataset is the seed list. Obstacles sit on a jittered grid chosen so the narrowest possible gap still exceeds the wingspan.

min_gap = grid_pitch - 2 * max_radius - 2 * max_jitter

= 2.50 - 2(0.30) - 2(0.25)  =  1.40 m      against a 1.00 m wingspan

cell centre    cx = -half + grid_pitch * (i + 0.5)

map projection px = (x + half) * MAP / room

py = MAP - (y + half) * MAP / room

Reachability was verified independently by flood filling the free space for a disc of the aircraft's radius. Every station was reachable at every obstacle density tested, which rules out an impossible course as an explanation for any failure.

11. MiDaS integration

11.1 What the model is and what it returns

MiDaS-small version 2.1 is an encoder and decoder network of 21.3 million parameters. The encoder is a lightweight convolutional backbone built for devices without a graphics processor. The decoder climbs back up, fusing coarse features that know the layout with fine features that know where edges are.

It is loaded from the published weights and never trained. What it returns is inverse depth, meaning one divided by distance, and only up to an unknown multiplier and an unknown offset that differ from image to image. It is not metres and it cannot be handed to the policy directly.

11.2 The preprocessing trap

The hub transform divides the image by 255 itself. Passing an image already scaled to the range zero to one therefore scales it twice, and the network sees a nearly black frame. It does not error. It returns a plausible looking vertical gradient for every input, which is easy to mistake for a domain gap.

| Measurement on simulator frames | Image scaled twice | Correct |
| --- | --- | --- |
| Correlation with true inverse depth | 0.29 | 0.92 |
| Absolute relative error | 0.40 | 0.21 |
| Pixels within a factor of 1.25 | 0.42 | 0.62 |

# correct

img = rgb.astype(np.float32)          # 0 to 255, the transform scales it

x   = transform(img)

# wrong, and it fails silently

img = rgb.astype(np.float32) / 255.0

The check that caught it was running the model on a real photograph. A featureless vertical ramp on a real indoor photo is not something any working monocular depth model produces, which located the fault upstream of the network rather than in it.

11.3 Recovering scale and offset

Both unknowns are found by least squares against a reference, in inverse depth space. Median scaling corrects the multiplier alone and leaves a systematic error behind, which is why the original project specification needed correcting on this point.

minimise over s and t:    sum_i ( s * pred_i + t - target_i )^2

# two unknowns, so a closed form 2x2 solve

a00 = sum(p*p)   a01 = sum(p)   a11 = N

b0  = sum(p*q)   b1  = sum(q)

det = a00*a11 - a01*a01

s   = ( a11*b0 - a01*b1) / det

t   = (-a01*b0 + a00*b1) / det

# then back to metres

depth = 1 / max( s * pred + t,  1e-3 )

Two alignment arms exist and they answer different questions.

| Arm | How scale and offset are obtained | What it measures |
| --- | --- | --- |
| frozen | Fitted once across a calibration set of simulator frames, then held fixed | The honest arm. It is what an aircraft could actually do, using a known height above the floor as its reference |
| oracle | Refitted on every frame against the true depth | An upper bound. It separates error in the depth structure from error in the alignment, and is not achievable in flight |

11.4 Where it plugs in

The policy takes the depth map as an argument and holds no dependency on how it was produced. That single seam is what makes the ablation a fair comparison, because everything downstream is byte for byte identical across the three arms.

from midas_depth import MidasDepth

md = MidasDepth()                       # picks Metal, CUDA or CPU automatically

md.calibrate(cal_rgb_frames, cal_true_depths)

def estimated(rgb, true_depth):

return md.metric_depth(rgb, true_depth, mode='frozen')

run(seed, stations, obstacles, room, depth_fn=estimated)

11.5 Quality metrics

ratio  = max( pred/true,  true/pred )

AbsRel = mean( |pred - true| / true )

RMSE   = sqrt( mean( (pred - true)^2 ) )

d1     = fraction of pixels with ratio < 1.25

d2     = fraction of pixels with ratio < 1.25^2

12. Measured values

Everything below was measured on this machine, an Apple M5 with 16 GB of memory. Nothing is quoted from the literature.

12.1 Speed

| Measurement | Value |
| --- | --- |
| MiDaS-small inference, Metal backend | 8.7 ms per frame, about 115 frames per second |
| Complete loop, physics with colour and depth render | over 200 cycles per second |
| Reactive policy computation alone | about 0.24 ms per call |
| Requirement to be met | 10 frames per second |

12.2 Depth quality on simulator frames

| World | Alignment | Correlation | AbsRel | d1 | d2 |
| --- | --- | --- | --- | --- | --- |
| flat grey | frozen | 0.917 | 0.222 | 0.598 | 0.902 |
| flat grey | oracle | 0.917 | 0.146 | 0.774 | 0.935 |
| textured | frozen | 0.915 | 0.207 | 0.622 | 0.918 |
| textured | oracle | 0.915 | 0.752 | 0.759 | 0.952 |

Texturing the world changed depth quality by less than one part in five hundred. Visual realism is therefore not what limits the estimate here, and the textures are kept because the simulator looks better rather than because they help the model.

12.3 Flight outcomes

Ground truth depth, fifty held out seeds never used during tuning, four stations per run.

| Obstacles in a 10 m room | Routes completed | Collision free | Contacts per run |
| --- | --- | --- | --- |
| 4 | 47 of 50 | 41 of 50 | 0.16 |
| 6 | 38 of 50 | 33 of 50 | 0.20 |
| 9 | 28 of 50 | 22 of 50 | 0.34 |

The three arm ablation on ten seeds at six obstacles, which is too small a sample to conclude from but shows the harness working end to end.

| Depth source | Routes completed | Collision free | Contacts per run |
| --- | --- | --- | --- |
| Perfect, from the simulator | 10 of 10 | 5 of 10 | 1.60 |
| MiDaS, from one camera image | 9 of 10 | 2 of 10 | 3.40 |
| None | 9 of 10 | 3 of 10 | 37.10 |

The contact column carries the signal. A blind aircraft touches something 37 times a run, one flying on an estimate 3.4 times, and one with a true sensor 1.6 times. One caution applies to every figure on this page. Tuning the policy on 12 seeds gave 92 per cent collision free, and the same settings gave 66 per cent on 50 seeds held back. Only the held out number should ever be quoted.

13. Parameter reference

Every value below is a constructor argument of the policy, which is what makes the automated parameter search the coursework asks for straightforward to run.

| Parameter | Default | Meaning |
| --- | --- | --- |
| max_range | 8.0 m | Beyond this a direction counts as fully open |
| stop_threshold | 1.2 m | Safety layer trip point. Linked to the stopping distance |
| release_margin | 0.0 m | Hysteresis on the stop layer. Tested and rejected |
| clear_distance | 3.5 m | Full speed once this much room is ahead |
| goal_weight | 1.8 | Pull toward the station in the heading score |
| commit | 0.25 | Penalty for abandoning the heading already chosen |
| half_span | 0.55 m | Half the wingspan plus margin. Sets the shadow width |
| z_band | 0.35 m | Height band around flight level that counts as an obstacle |
| min_throttle | 0.30 | Speed floor, so it commits or declares itself blocked |
| smoothing | 0.25 s | Time constant of the heading average |
| memory_bins | 72 | Bearing bins in the all round memory, one every 5 degrees |
| forget | 0.15 m/s | Memory decay. The most influential single parameter |
| escape_after | 1.0 s | Time blocked before backing out |
| escape_speed | 0.35 m/s | Reverse speed while escaping. Blind, so it is slow |
| desperate_after | 3.0 s | Time blocked before the station is ignored entirely |
| flip_cooldown | 0.0 s | Wing guard side switching. Tested and rejected |
| max_speed | 2.0 m/s | Cruise speed the latency budget was derived from |
| max_yaw | 1.8 rad/s | Yaw rate limit |
| max_climb | 1.0 m/s | Climb rate limit |

Airframe and world constants, taken from the team's own design files rather than assumed.

| Constant | Value | Source |
| --- | --- | --- |
| Wingspan | 1.000 m | Measured from 09_vtol_shell_combined_aero.step |
| Length | 0.815 m | Same file |
| Height | 0.231 m | Same file |
| Camera vertical field of view | 100 degrees | Chosen by measurement, not by default |
| Camera resolution | 320 by 240 | What the depth model consumes |
| Room | 10 m square, 3 m high | Set by the wingspan |
| Narrowest obstacle gap | 1.40 m | Leaves 0.20 m clearance each side |
| Cruise altitude | 1.20 m | Mid height between floor and ceiling |
| Control step | 0.05 s | 20 cycles per second |